程式：WebServerControlRelay 使用Client模式建立網頁伺服器

開啟程式WebServerControlRelay

程式位址：https://github.com/brucetsao/BruceCourses/blob/master/105ANQU_IOT/Code/WebServerControlRelay/WebServerControlRelay.ino

#include <WiFi.h> char ssid[] = "Apple_TC_Wi-Fi"; // your network SSID (name) char pass[] = "27541147"; // your network password int keyIndex = 0; // your network key Index number (needed only for WEP) int status = WL_IDLE_STATUS; WiFiServer server(80); #define RelayPin 8 #define Turnon HIGH #define Turnoff LOW void setup() { Serial.begin(9600); // initialize serial communication pinMode(RelayPin, OUTPUT); // set the LED pin mode // check for the presence of the shield: if (WiFi.status() == WL_NO_SHIELD) { Serial.println("WiFi shield not present"); while (true); // don't continue } String fv = WiFi.firmwareVersion(); if (fv != "1.1.0") { Serial.println("Please upgrade the firmware"); } // attempt to connect to Wifi network: while (status != WL_CONNECTED) { Serial.print("Attempting to connect to Network named: "); Serial.println(ssid); // print the network name (SSID); // Connect to WPA/WPA2 network. Change this line if using open or WEP network: // status = WiFi.begin(ssid); //no pass status = WiFi.begin(ssid, pass); //wpa use // wait 10 seconds for connection: delay(10000); } server.begin(); // start the web server on port 80 printWifiStatus(); // you're connected now, so print out the status } void loop() { WiFiClient client = server.available(); // listen for incoming clients if (client) { // if you get a client, Serial.println("new client"); // print a message out the serial port String currentLine = ""; // make a String to hold incoming data from the client while (client.connected()) { // loop while the client's connected if (client.available()) { // if there's bytes to read from the client, char c = client.read(); // read a byte, then Serial.write(c); // print it out the serial monitor if (c == '\n') { // if the byte is a newline character // if the current line is blank, you got two newline characters in a row. // that's the end of the client HTTP request, so send a response: if (currentLine.length() == 0) { // HTTP headers always start with a response code (e.g. HTTP/1.1 200 OK) // and a content-type so the client knows what's coming, then a blank line: client.println("HTTP/1.1 200 OK"); client.println("Content-type:text/html"); client.println(); // the content of the HTTP response follows the header: client.print("Click <a href=\"/H\">here</a> turn Power on<br>"); client.print("Click <a href=\"/L\">here</a> turn Power off<br>"); // The HTTP response ends with another blank line: client.println(); // break out of the while loop: break; } else { // if you got a newline, then clear currentLine: currentLine = ""; } } else if (c != '\r') { // if you got anything else but a carriage return character, currentLine += c; // add it to the end of the currentLine } // Check to see if the client request was "GET /H" or "GET /L": if (currentLine.endsWith("GET /H")) { digitalWrite(RelayPin, Turnon); // GET /H turns the LED on } if (currentLine.endsWith("GET /L")) { digitalWrite(RelayPin, Turnoff); // GET /L turns the LED off } } } // close the connection: client.stop(); Serial.println("client disonnected"); } } void printWifiStatus() { // print the SSID of the network you're attached to: Serial.print("SSID: "); Serial.println(WiFi.SSID()); // print your WiFi shield's IP address: IPAddress ip = WiFi.localIP(); Serial.print("IP Address: "); Serial.println(ip); // print the received signal strength: long rssi = WiFi.RSSI(); Serial.print("signal strength (RSSI):"); Serial.print(rssi); Serial.println(" dBm"); // print where to go in a browser: Serial.print("To see this page in action, open a browser to http://"); Serial.println(ip); }

WebServerControlRelay程式重點解說

- #include <WiFi.h>  使用網路必要函數

- WiFiServer server(80);   啟動PORT 80進行

- status = WiFi.begin(ssid);		不使用加密連AP

- status = WiFi.begin(ssid, pass);	使用加密連AP

- server.begin(); 	開始啟動PORT 80 傾聽

- printWifiStatus();	列印網路資訊

- WiFiClient client = server.available(); 有人連接Port 80

- client.connected()  有用戶連接中

- client.available()	用戶送資料進來

- char c = client.read();	讀出用戶送的資料(一個位元組)

- client.println(“HTTP/1.1 200 OK”); ….  送給用戶端一段HTML碼，用瀏覽器方能顯示

- currentLine.endsWith(“GET /H”)  判斷是否用/H 結束

- currentLine.endsWith(“GET /L”)  判斷是否用/L 結束

- client.stop();	與用戶連線停止傳輸資料