程式：Ameba Blink程式

開啟程式Blink

程式位址：

/* Blink Turns on an LED on for one second, then off for one second, repeatedly. Most Arduinos have an on-board LED you can control. On the Uno and Leonardo, it is attached to digital pin 13. If you're unsure what pin the on-board LED is connected to on your Arduino model, check the documentation at http://www.arduino.cc This example code is in the public domain. modified 8 May 2014 by Scott Fitzgerald */ // the setup function runs once when you press reset or power the board void setup() { // initialize digital pin 13 as an output. pinMode(13, OUTPUT); } // the loop function runs over and over again forever void loop() { digitalWrite(13, HIGH); // turn the LED on (HIGH is the voltage level) delay(1000); // wait for a second digitalWrite(13, LOW); // turn the LED off by making the voltage LOW delay(1000); // wait for a second }

簡單來說，Blink 這個讓 LED 燈泡閃爍，它使用的GPIO接腳是13
Ameba的Pin一樣在D13的位置，根據Ameba的接腳位置圖：

於是我們將電阻以及LED燈泡連接如下圖：
（NOTE：LED裡，長腳的是正極，接到D13的地方，短腳的是負極，接到GND的地方）

然後按一下Reset按鈕，就可以看到燈炮在閃爍，這個實驗就完成了。
如果以上過程遇到問題，請參考 Trouble-shooting  看是否能解決。

=========================================

CheckAP程式重點解說

pinMode(13, OUTPUT);’

digitalWrite(13, HIGH);

digitalWrite(13, LOW);